United States Dept. Of Transportation

Office of Pipeline Safety

Public Meeting on

Reconsideration of Maximum Allowable Operating Pressure in Natural Gas Pipelines

March 21, 2006

“Panel on Perspectives from State Pipeline Safety Programs on Preserving Safety”

Remarks by

Gary Farmer

The Maritimes and Northeast Pipeline has petitioned OPS to waive Part 192 MAOP regulations for Class 1, 2 and 3 on existing and proposed pipelines.

1.  Pipe Integrity Improvements since the establishment of 72% SMYS in the 1950s

- Metallurgical and Technological Advances

Metallurgy – Fracture Toughness, Ductility, Critical Defect Size, Hydro test Level and Duration, Mill Quality Control

Welding – Automatic, Ultrasonic Inspection

Coatings – Fusion Bonded Epoxy

- Comprehensive Threat Analysis

External and Internal Corrosion

Stress Corrosion Cracking

Manufacturing & Construction Defects

Excavation Damage (3rd party)

Human Error

- Integrity Management

In Situ Inspection Technologies Now Available – MFL, etc. Tools, Radiographic and Ultrasonic, Weld Inspections

Life Cycle Management of Pipe

Operator Qualification Program (address human error)

Damage Prevention Program (3rd party damage)

2.  Risk = Probability of the Occurrence Times Consequences of the Occurrence

- Threat Analysis Only Addresses Probability

Probability is never zero, so consequences must be considered

- Increased MAOP Increases Consequences

Current waiver petition includes existing pipeline - Class 2 to 67%, Class 3 – 56%

This Meeting only addressed Class 1 to 80%

There will be no record on the advisability of increasing MAOP in Class 2 and Class 3 where consequences are greater.

- Unknown Stresses – Will they still be adequately addressed?

Residual construction stresses, uneven externally-applied loads

- Grandfathering of MAOP Due to Class Location Changes

Class 2 (60%) from Class 1 (72%) (with waiver 80%)

Class 3 (50%) from Class 2 (60%) (with waiver 67%)

- Current Regulations Which Allow Grandfathering, Do Not Address Consequences            Consequences will increase again with waivers - “Double dipping” of the Integrity Management Protocols?

- Integrity Management and OQ Regulations Have Not Been in Place Long Enough to Demonstrate Effectiveness.

3.  In Summary

- Develop An Independent Analysis Of The Probability Of Threats To The Pipeline Occurring As The Result Of Increased MAOP – Credibility with the Public

- Expand The Analysis to Include the Consequences of  All Class Locations                 (1 Through 4)

- Rulemaking vs. Waiver Petition

Either way should require:

A complete risk analysis;

An expanded public education program for the parties directly impacted – should discuss   risks and consequences; and

An Assessment of the Credibility of the operator – inspection and compliance history, integrity management history.

For intrastate gas transmission lines, a rulemaking could limit state regulatory agency oversight and conditions.